4 Channels UHF RFID Fixed Reader                                                                  OPM6A

Max Tag Read Rat：:More than 750 tags/second using high-performance settings

Working mode：the principal andsubordinate mode /off-line working mode/Trigger mode

In the hardware: the clock circuit is designed

Interface：

1.The synopsis of the reader：

The OPM6A is a low profile, high-performance 4-port RFID reader designed for both indoor and outdoor applications. Driven byThingMagic’spowerful Mercury6e UHF RFID reader module.

OPM6A has built-in Linux operating system, which not only meets the complex customer business requirements, but also responds to various label operations independently and quickly.

2.Product features

| Mechanical electrical performance |  |
| --- | --- |
| Dimension：（MM） | 215（L）*158（W）*38（H） |
| Net Weight ：（KG） | 1.18±0.05KG |
| Gross weight：(KG) | 1.176±0.05KG |
| Performance Index |  |
| Tag / Transponder Protocols | EPCglobalGen2(ISO18000-6C) with DRM ISO18000-6B(optional) IP-X(optional) |
| RF Connector | 4 RP-TNC antenna connectors |
| Frequency | Pre-configuredforthefollowingregions: FCC 902-928MHz(Americas) ETSI 865.6-867.6MHz(EU) KCC 917-920.8MHz(Korea) TRAI 865-867MHz(India) ACMA 920-926MHz(Australia) ‘Open’ (Customizable)865-869MHz, 902-928MHz |
| RF output | Separate read and write levels,command adjustable from 5dBm to31.5 dBm* (1.4W) with +/-0.5dBm accuracy above +15dBm |
| Reading distance | Over 30 feet(9m) with 6dBi antenna (36dBm EIRP) |
| Peak inventory speed | > 750 tags/sec |
| Tag Buffer Size | ＞100,000tags @ 96 bit EPC |
| Tag RSSI | Supported |
| Antenna Detector | Supported |
| Working mode | the principal and subordinate mode /off-line working mode/Trigger mode |
| Power | With 220V AC input, the output of +12 V/3A DC power converter |
| input / output (GPIO) | 3 inputs, 3 output |
| ommunication interface | 10M/100MAdaptive Ethernet、RS232 |
| Operating System | Linux |
| Application Interface | Direct Communication:EPCglobal Low Level Reader Protocol (LLRP) v1.1 On reader API:MercuryOS CAP I HostAPI:Java,C,.NET |
| Environment |  |
| Operating Temp | -20～+55℃ |
| Storage Temp | -20～+85℃ |
| Humidity | 5%～95%RH，Non-condensing |