
[image: image1.png]



 UHF RFID Integrated Reader   

                          OPP-UA3
1.The synopsis of the reader：                                                                  

OPP-UA3 is a high-performance UHF RFID integrated Reader, It combines a proprietary, efficient signal processing algorithm that maintains a high identification rate while achieving rapid read/write processing of  RFID tags. Its flexible multi-interface design ensures seamless interoperability between interfaces, it's widely used in automobile manufacturing,, equipment manufacturing, logistics sorting, workstation assembly lines, tool management, AGV robot control and other scenarios.

2.Product features 
	Mechanical electrical performance

	Equipment size  Dimension：（MM）
	95（L）*95(W)*42（H）（±2）

	Diemension including packaging（MM）
	255（L）*200（W）*70（H）（±5）

	Net weight：    (KG)
	0.39±0.01

	Gross weight：（KG）
	1.20±0.01

	Shell
	Surface shell: ABS, bottom shell: aluminum alloy

	RFID Chip
	Impinj E310

	Antenna
	Ceramic antenna with a gain greater than 2DB (can be optionally equipped with an antenna according to requirements, such as a near-field antenna)

	Performance Index

	Tag / Transponder Protocols
	EPCglobal C1 Gen 2 ISO 18000-6C 

	Frequency
	865~868MHz/902～928MHz（Can be adjusted according to different countries or regions）

	RF output
	from 0 dBm to 26dBm* with  +/-1dBm

	Reading distance
	＞2m（eg.9662，depends on tag）

	Peak inventory speed
	>100 tags/second

	Tag RSSI
	Supported

	Antenna Detector
	Supported

	Working mode
	the principal and subordinate mode /off-line working mode/Trigger mode

	Power
	With 220V AC input, the output of +12 V/4A DC power converter、POE（Optional），

	Relay interface
	see 5.1 for details

	Input / Output (GPIO)
	see 5.2 for details

	Indicator light
	Red LED:Power indicator

Green LED:Tag indicator light

Yellow LED:Reading  indicator

	Communication interface
	RJ45, RS232

	Operating System
	Linux 

	Communication protocol
	RS-232, TCP/IP, Modbus (Optional), HTTP/HTTPS (Optional), MQTT (Optional), RabbitMQ (Optional)

	Host  API
	C#、JAVA

	Development Environment
	Linux、Android、Windows、PLC、Web

	Environment

	Operating Temp
	-20～+55℃

	Storage Temp
	-20～+85℃

	Humidity
	5%～95%RH，Non-condensing


3.MECHANICAL DATA (UNIT:mm) 
[image: image2.png]95.0 -

3.0

—42.0—=

80.0

ﬁx Mapeci

8.8

- M4 2L



                                      

 Figure2

4.The product image:     
 

[image: image3.png]


[image: image4.png]Relay
see 5.1 for details

GPIO
see 5.2 for details|





Figure3                                        Figure4
5.Interface Description

[image: image5.png]



                  Figure 5
5.1 Definition of Relay
	Pin 1
	Relay1

	Pin 2
	

	Pin 3
	Relay2

	Pin 4
	


5.2GPIO definition description

	Pin 1
	GPI-

	Pin 2
	GND


6.ACCESSORY
	Name
	Description
	Quantity

	Power Adapter
	12V/4A，Power Adapter+a Power Cord with a length of 1.5 meters 
	1

	mounting bracket
	Standard configuration includes L-shaped bracket
	1


[image: image6.jpg]


[image: image7.jpg]



Figure 6 Power Adapter*1     Figure 7 Power Cord*1     Figure 8 L-shaped bracket
Product upgrades or specification adjustments will not be notified separately.

