4 Channels UHF RFID Fixed Reader                                                                  SRE-401

Peak inventory speed：> 700 tags/sec

Working mode：the principal andsubordinate mode /off-line working mode/Trigger mode

In the hardware: the clock circuit is designed

Interface：

1.The synopsis of the reader：

The SRE-401 is a low profile, high-performance 4-port RFID reader designed for both indoor and outdoor applications. Adopt Impinj IndyR2000 chip, with high-performance multi-tag identification algorithms.

SRE-401 has built-in Linux operating system, which not only meets the complex customer business requirements, but also responds to various label operations independently and quickly. Multi-tag reading performance is excellent, supporting intensive reading, large capacity tag reading and writing, received signal strength detection (RSSI). It has reliable network adaptability, supports a variety of network protocols, and is easy to deploy large-scale applications.

2.Product features

| Mechanical electrical performance |  |
| --- | --- |
| Dimension：（MM） | 215（L）*158（W）*38（H） |
| Weight：（KG） | 1.24±0.05KG |
| Performance Index |  |
| Tag / Transponder Protocols | EPCglobal Gen 2 (ISO 18000-6C)、ISO 18000-6B (optional) |
| RF Connector | 4 RP-TNC antenna connectors |
| Frequency | 865~868MHz/902～928MHz（Can be adjusted according to different countries or regions） |
| Frequency modulation | FHSS or fixed-frequency, can be set by software |
| RF output | from 0 dBm to 30 dBm* with +/-1dBm |
| Reading distance | 9dbi Antenna configuration, typical read range: 15 to 25 m (depends on tag) |
| Peak inventory speed | > 700 tags/sec |
| Tag Buffer Size | 100,000tags @ 96 bit EPC |
| Tag RSSI | Supported |
| Antenna Detector | Supported |
| Working mode | the principal and subordinate mode /off-line working mode/Trigger mode |
| Power | With 220V AC input, the output of +12 V/5A DC power converter |
| input / output (GPIO) | 3 inputs, 3 output |
| ommunication interface | 10M/100MAdaptive Ethernet、RS232 |
| Operating System | Linux |
| Environment |  |
| Operating Temp | -20～+55℃ |
| Storage Temp | -20～+85℃ |
| Humidity | 5%～95%RH，Non-condensing |