OPP-D09

1-Port Integrated UHF RFID Reader

Peak inventory speed：> 700 tags/sec

Working mode：the principal andsubordinate mode /off-line working mode/Trigger mode

In the hardware: the clock circuit is designed

1.The synopsis of the reader：

OPP-D09 is an easy to install, unobtrusive, integrated 1-port reader/antenna designed for office and commercial environments. Adopt Impinj IndyR2000 chip, with high-performance multi-tag identification algorithms.

2.Product features

| Mechanical electrical performance |  |
| --- | --- |
| Dimension：（MM） | 260（L）*260（W）100（H） |
| Weight：（KG） | 2.1KG |
| Performance Index |  |
| Tag / Transponder Protocols | EPCglobal Gen 2 (ISO 18000-6C)、ISO 18000-6B (optional) |
| Antenna | 9DB circular polarization |
| Frequency | 865~868MHz/902～928MHz（Can be adjusted according to different countries or regions） |
| Frequency modulation | FHSS or fixed-frequency, can be set by software |
| RF output | from 0 dBm to 30 dBm* with +/-1dBm |
| Reading distance | 9dbi Antenna configuration, typical read range: 15 to 25 m (depends on tag) |
| Peak inventory speed | > 700 tags/sec |
| Tag Buffer Size | 200,000tags @ 96 bit EPC |
| Tag RSSI | Supported |
| Antenna Detector | Supported |
| Working mode | the principal andsubordinate mode /off-line working mode/Trigger mode |
| Power | With 220V AC input, the output of +12 V/3A DC power converter |
| input / output (GPIO) | 1 inputs, 1 output |
| ommunication interface | 10M/100MAdaptive Ethernet、RS232 |
| Operating System | Linux |
| Environment |  |
| Operating Temp | -20～+55℃ |
| Storage Temp | -20～+85℃ |
| Humidity | 5%～95%RH，Non-condensing |