UHF rfid integrated reader

OPP-D01

- USB drive free (windows, Android)

- It has the function of audible and visual alarm

1. Product introduction:

OPP-D01 is a high-performance UHF RFID desktop integrated reader,based on the design of high-performance UHF chip, it fully supports the EPCglobal C1 Gen 2 ISO 18000-6c standard,Combined with the proprietary high-efficiency collision processing algorithm, while maintaining a high reading rate, it can realize the fast reading and writing processing of electronic tags, which can be widely used in various solutions such as personal identification, conference sign in system, access control system, anti-counterfeiting system and production process control.

- 2.Product parameters：

| Mechanical electrical performance |  |
| --- | --- |
| Equipment size Dimension：（MM） | 170（L）*122(W)*20(H)MM（±1） |
| Diemension including packaging（MM） | 250（L）*230(W)*50(H)MM（±1） |
| Net weight：(KG) | 0.22（±0.02） |
| Gross weight：（KG） | 0.38（±0.02） |
| Antenna | Integrated Near field circularly polarized antenna |
| Shell | Metal frame, ABS surface shell |
| Performance Index |  |
| Tag / Transponder Protocols | EPCglobal C1 Gen 2 ISO 18000-6C |
| Frequency | 865~868MHz/902～928MHz（Can be adjusted according to different countries or regions） |
| Frequency modulation | FHSS or fixed-frequency, can be set by software |
| RF output | from 0 dBm to 26dBm* with +/-1dBm |
| Reading speed | >50 tags/sec |
| Tag Buffer Size | 330 tags @ 96 bit EPC/100@ 96 bit EPC |
| Tag RSSI | Supported |
| Interface: | Communication Interface USB drive free type-C interface |
| Indicator light | PWR: Red LED, Power indicator COM: Green LED , Communication indicator TAG： Blue LED ， Tags operation indicator |
| API Support | C++，C#，Java，NET |
| Certification | ROHS、FCC |
| - Environment |  |
| Operating Temp | -20～+55℃ |
| Storage Temp | -20～+85℃ |
| Humidity | 5%～95%RH，Non-condensing |

- Product images

Figure 2

Figure3

- OPP-D01 Family Series:

Figure 4

The above is the OPP-D01 standard version, with two additional specifications of the same size:

1. OPP-D01-K, HID mode, automatically simulates keyboard output for read tag data

2. OPP-D01-T, used for tag detection. According to the set reading threshold, after successful reading, the device's green light will light up and the buzzer will sound（See the figure below“Figure 5.

”）

Figure 5

- Dimension drawing （MM）

Figure 6

- Packing List

The OPP-D01 Reader * 1, USB data cable 1.5 meters * 1, Certificate of conformity * 1